Introduction to Robotic Systems Course

LAB 2

Interrupt Lab Exercise:

Stack Use and Timing Behavior

- Contents

1	Introduction	1

1.1	Lab Overview	1

2	Requirements	1

3	Task: Software	1

4	Task: Hardware Setup	1

5	Task: Analysis	3

5.1	CPU Behavior	3

5.1.1	CPU State when Entering HANDLER	3

5.1.2	CPU State after Entering HANDLER	3

5.1.3	CPU State after Exiting Interrupt	4

5.2	Timing	4

5.2.1	Observe Overall CPU Timing Behavior	4

5.2.2	Observe Detailed CPU Timing Behavior	4

5.2.3	Observe Even More Detailed CPU Timing Behavior	5

5.2.4	Pre-emption of Main Code	5

# Introduction

## Lab Overview

This lab exercise presents an example code to introduce the concept and application of interrupts in micro-controller operation using an interrupt service routine (ISR) to service interrupts received from a dual in-line package (DIP) switch. In the code supplied for this lab, the ISR increments a counter whenever an interrupt is received. LEDs connected to the micro-controller output the state of the counter. Also, in this lab, the states of the processor during context switching and interrupt execution is examined using the Keil debug tool.

# Requirements

The following hardware and software are required to complete this lab:

- Hardware

  - OpenCR1.0 Microcontroller Board and ULINK-ME debugger

  - Dip Switch, LEDs (x5), 270Ω Resistors (x3), Breadboard and jumper cables

  - Logic analyzer

- Software: Keil MDK. Version v5.28 (course was developed with this version)

# Task: Software

We have provided the complete source code for this lab.

# Task: Hardware Setup

Use Table 1, the schematic shown in Figure 1, and the OpenCR1.0 board layout in Figure 2 as a guide to connect the switch, LEDs, and resistors to the GPIO port on the MCU.

Connect the debug signals (DBG_Main and DBG_ISR) and the switch signal to a logic analyzer or oscilloscope. The signal DBG_ISR will indicate execution of the handler, while the signal DBG_Main will indicate the program is out of handler mode and is in a loop.

Table 1: Signals and connections

| Signal Name | Description | Direction | MCU | Arduino Pin on OpenCR1.0 |
| --- | --- | --- | --- | --- |
| SW1 | Switch Input | Input to MCU | PB4 | 3 |
| DBG_Main | Main Thread Debug Output | Output from MCU | PC1 | 7 |
| DBG_ISR | ISR Debug Output | Output from MCU | PA2 | 6 |
| Red LED | Counter state | Output from MCU | PA8 | 5 |
| Green LED | Counter state | Output from MCU | PC6 | 1 |
| Blue LED | Counter state | Output from MCU | PC7 | 0 |

Figure 1: Schematic of LEDs and switch connected to the OpenCR1.0.

Figure 2: Layout of OpenCR1.0 Highlighting Arduino Pins.

# Task: Analysis

- Open the supplied project folder and run the Keil MDK project.

- Compile and load the code onto the board.

- Start the debugger session.

- Enable the disassembly window (View->Disassembly Window)

- Set a breakpoint at start of handler function.

- Run the program and then press the switch SW1

## CPU Behavior

### CPU State when Entering HANDLER

Examine the stack and CPU registers with the debugger.

- Complete the table below to show the values of the CPU registers and state information.

| Register | Value | Register | Value | Register/State | Value |
| --- | --- | --- | --- | --- | --- |
| R0 | – | R8 | – | xPSR | – |
| R1 | – | R9 | – | MSP | – |
| R2 | – | R10 | – | PSP | – |
| R3 | – | R11 | – | PRIMASK | – |
| R4 | – | R12 | – | CONTROL | – |
| R5 | – | R13 (SP) | – | Mode | – |
| R6 | – | R14 (LR) | – | Privilege | – |
| R7 | – | R15 (PC) | – | Stack | – |

- Complete the table below to show what information is on the stack. Open a memory window (View->Memory Windows->Memory 1) and enter SP as the address. Right-click on the window and specify Unsigned->Int as the display format.

| Address | Value | Description |
| --- | --- | --- |
| (SP) | – | – |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |

### CPU State after Entering HANDLER

Step one line with F11, then examine the stack and CPU registers with the debugger.

- Complete the table below to show the values of the CPU registers and state information.

| Register | Value | Register | Value | Register/State | Value |
| --- | --- | --- | --- | --- | --- |
| R0 | – | R8 | – | xPSR | – |
| R1 | – | R9 | – | MSP | – |
| R2 | – | R10 | – | PSP | – |
| R3 | – | R11 | – | PRIMASK | – |
| R4 | – | R12 | – | CONTROL | – |
| R5 | – | R13 (SP) | – | Mode | – |
| R6 | – | R14 (LR) | – | Privilege | – |
| R7 | – | R15 (PC) | – | Stack | – |

- Complete the table below to show what information is on the stack. Open a memory window (View->Memory Windows->Memory 1) and enter SP as the address. Right-click on the window and specify Unsigned->Int as the display format.

| Address | Value | Description |
| --- | --- | --- |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |
| – | – | – |

### CPU State after Exiting Interrupt

Step out of the handler function (ctrl + F11) and verify the return address.

## Timing

- Now connect the debug signals to a logic analyzer or oscilloscope if you have not done so already.

- Disable the breakpoint in the handler function and any other breakpoints you may have added.

- Resume program execution.

### Observe Overall CPU Timing Behavior

Use the falling edge of the Switch input to trigger the data capture/sweep.

Set the time base of the logic analyzer (or oscilloscope) so that a switch press covers about one fourth of the screen. Capture a screenshot showing the switch signal, DBG_ISR, and DBG_MAIN.

- How long was the switch pressed down?

- Is there any noticeable delay between the switch being pressed and the ISR running?

- Does the DBG_MAIN indicate that main stops running at any time?

### Observe Detailed CPU Timing Behavior

Now zoom in so that the screen displays about 100 µs, centered on the ISR.

- How long is the DBG_ISR signal asserted?

- Is there any noticeable delay between the switch being pressed and the ISR running?

- Does the DBG_MAIN signal indicate that main stops running at any time?

### Observe Even More Detailed CPU Timing Behavior

Now zoom in so that the screen displays about 10 µs centered on the ISR.

- How long is the DBG_ISR signal asserted?

- Is there any noticeable delay between the switch being pressed and the ISR running? How many clock cycles does this correspond to? How does this compare to what you would expect?

- Does the DBG_MAIN signal indicate that main stops running at any time? If so, calculate for how long.

### Pre-emption of Main Code

Now zoom in so that the screen displays about 100 us centered on the ISR.

- For how long is the main function delayed? First measure the pulse width of the DBG_MAIN output signal before the switch is pressed. Then, measure the pulse width when main is pre-empted by the ISR. The difference indicates the total pre-emption time.

- How long is the total pre-emption in comparison with the duration of the DBG_ISR signal? If the two times aren’t the same, explain why.